3GPP TSG-SA WG3 LI	Tdoc S3LI02_084r2

Orlando, Florida. 09 – 11 April 2002

| CR-Form-v4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 33.106 |  |  |  | CR | CRNum |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 5.1.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  | x |
| Title:  |  | Changes to 33.106 to clarify interception capabilities. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Telcordia Technologies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 9 April 2002 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | Rel-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | The reference to J-STD-025 is updated to a later revision. In addition, include enhancements and clarification regarding requirements for encryption of intercepted communications, unobtrusiveness of interception, and integrity of intercepted data. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Clarify the handling of interception (e.g., unobtrusiveness) and update to a Section 2 references. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Misalignment with J-STD-025-A. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 2, 5.2.1.3, and 5.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  | This CR has been agreed to by T1P1.SAH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.

[1]	European Union Council Resolution on the Lawful Interception of Telecommunications (17. January 1995)

[2]	ETR 331: "Definition of User Requirements for Lawful Interception of Telecommunications; Requirements of the Law Enforcement Agencies".

[3]	ES 201 158: "Lawful Interception; Requirements for network functions".

[4]	ES 201 671: "Handover Interface for the lawful interception of telecommunications traffic".

[5]	GSM 01.33: "Lawful Interception requirements for GSM".

[6]	GSM 02.33:" Lawful Interception - stage 1".

[7]	GSM 03.33: "Lawful Interception - stage 2".

[8]	J-STD-025-A, "Lawfully Authorised Electronic Surveillance".

#### 5.2.1.3	Security of processes

The intercept function shall only be accessible by authorised personnel.

To be effective, interception must take place without the knowledge of either party to the communication. Therefore, decryption must also take place without either party being aware that it is happening.

No indication shall be given to any person except authorised personnel that the intercept function has been activated on a target. Authentication, encryption, audits, log files and other mechanisms may be used to maintain security in the system. Audit procedures should be capable of keeping accurate logs of administration commands.

The interception functions shall be implemented in such a manner that:

- the interception subject and his correspondents can not know that a lawful interception is active;

- during the intercepted communication itself the quality of the communication shall remain the same as usual and the service shall be unchanged, including all supplementary services such as call forwarding, etc.;

- when there is no intercepted communication the quality of the communication shall remain the same as usual and the service shall be unchanged, such that there is no modification to services supplied or information received either by the interception subject or by some other party.

Only specifically authorized personnel should be able to control interception.

For prevention of unauthorized administration, as well as unauthorized use, appropriate security features are necessary.

A security management system should be established.

There should be physical and logical access controls.

Any necessary keys, passwords and user identifications for the authorization and the logical access to the Interception function should be securely stored.

Any transmission of passwords and user identifications for access to interception functions should be secure.

Physical interfaces should be secured mechanically and/or logically against unauthorized use.

The network operator, access provider or service provider shall, in relation to each target service:

a)    remove any service coding or encryption which has been applied to the content of communication (i.e. en clair) and the intercept related information at the instigation of the network operator or service provider;

NOTE 1: If coding/encryption cannot be removed through means, which are available in the network or service for the given communication, the receiving agencies should be provided with keys etc. to access the information en clair, cf next clause.

b)    provide the LEA with any other decryption keys whose uses include encryption of the content of communication, where such keys are available for NWO/SvP/AP;

## 5.7 Security Aspects

A secure communication channel has the following properties:

- Confidentiality;

- Integrity;

- Authentication;

- Availability.

Confidentiality means that it is impossible to interpret the data by eavesdropping on the communication link.

Integrity means that any alteration or mutilation of the transported data is immediately detected.

Authentication means that the communicating parties have verified and confirmed each other's identities.

Availability means that the communicating parties have made agreements about up- and downtimes of the systems.

In case of irregularities, alarm messages should be sent through another communication channel. Because of the nature of the transported data, confidentiality can be an issue. Lawful Intercepted data can also be confidential or secret by law and appropriate measures need to be taken to prevent eavesdropping by unauthorized third parties.

5.7.1 Security Mechanisms

This clause will give an overview of possible mechanisms to achieve the properties as described above.

Technical details are to be decided during the process of implementation.

Confidentiality can be achieved by using encryption. A common technique is to use a symmetric encryption algorithm.

A symmetric algorithm is an algorithm where both communicating parties use the same key for encryption and decryption. This key must be exchanged in a secure way.

Integrity can be achieved using hashing algorithms. These algorithms generate a unique fingerprint of the transported data. When the transported data is altered, the fingerprint does not match anymore and appropriate actions can be undertaken (like retransmission of data).

Authentication can be achieved using cryptographic techniques. A common technique is to use asymmetric encryption.

In this technique, both parties have two keys: A public key and a private key. Data encrypted with one key can only be deciphered with the other. If party X encrypts something using the public key of party Y then party Y is the only one able to decrypt this data using his private key. If party X encrypts something using his private key then this data can only be deciphered using his public key. By combining these properties, both parties can make sure that they are communicating with the right party.